Boss SD-1 Super Overdrive to Ibanez TS808 Tubescreamer Conversion

This was fairly easy to do though I must stress, this DOES NOT make this pedal a TS808 in any way. There are several more part differences than these between the two. The biggest difference between the TS variants and the SD-1 is the diode clipping. Ibanez uses symmetrical and Boss uses asymmetrical. Remove D6 and put a jumper in its place to make the SD-1 have symmetrical clipping. D4 and D5 are a pair of the stock silicon diodes.

Some capacitors need to be adjusted.

· C1 change from 0.047uf to 0.02uf (0.022uf will work fine here too).

· C2 change from 0.018uf to 1uf NP (non-polarity).

· C4 change from 0.018uf to 0.22uf (I put two 0.1uf caps in parallel to get as close as I could, 0.2uf).

· C6 remove.

· C7 change from 1uf electrolytic to 1uf NP (non-polarity).

· C8 change from 0.047uf to 0.1uf.

· C10 change from 1uf to 10uf.

Next are a few resistor changes.

· R1 change from 10k to 1k.

· R2 change from 470k to 510k.

· R4 change from 100k to 10k.

· R5 change from 33k to 51k.

· R7 change from 10k to 1k.

· R10 change from 4.7k to 1k.

· R16 change from 1k to 100 ohms.

· R17 change from 100k to 10k.

And lastly, if you wish, change the LED to green from the stock red. It’s easier than repainting the whole enclosure. It’s just my own little personal touch. Everybody else uses blue or purple or some other color. So, I picked green.

Again, these changes don’t make the SD-1 a TS-808, but they do get it in the ballpark and give it that flavor, which is what I was aiming for.

